# Week 7 Lab – Configuring Basic Single-Area OSPFv2

- Topology

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | F0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | F0/1 | 192.168.13.1 | 255.255.255.252 | N/A |
|  | S0/0/0 (DCE) | 192.168.12.1 | 255.255.255.252 | N/A |
| R2 | F0/0 | 192.168.2.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.12.2 | 255.255.255.252 | N/A |
|  | S0/0/1 (DCE) | 192.168.23.1 | 255.255.255.252 | N/A |
|  | Lo0 | 209.165.201.2 | 255.255.255.255 | N/A |
| R3 | F0/0 | 192.168.3.1 | 255.255.255.0 | N/A |
|  | F0/1 | 192.168.13.2 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.23.2 | 255.255.255.252 | N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC-B | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 |

- Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure and Verify OSPF Routing

Part 3: Configure and Redistribute a Default Route

Part 4: Change Router ID Assignments

Part 5: Configure OSPF Passive Interfaces

- Background / Scenario

Open Shortest Path First (OSPF) is a link-state routing protocol for IP networks. OSPFv2 is defined for IPv4 networks, and OSPFv3 is defined for IPv6 networks. OSPF detects changes in the topology, such as link failures, and converges on a new loop-free routing structure very quickly. It computes each route using Dijkstra’s algorithm, a shortest path first algorithm.

In this lab, you will configure the network topology with OSPFv2 routing within the single Area Area 0, change the router ID assignments, configure passive interfaces, adjust OSPF metrics, and use a number of CLI commands to display and verify OSPF routing information.

- Build the Network and Configure Basic Device Settings

In Part 1, you set up the network topology and configure basic settings on the PC hosts and routers.

  - Cable the network as shown in the topology.

  - Initialize and reload the routers as necessary.

  - Configure basic settings for each router.

    - Disable DNS lookup.

    - Configure device name as shown in the topology.

    - Assign class as the privileged EXEC password.

    - Assign cisco as the console and vty passwords.

    - Configure a message of the day (MOTD) banner to warn users that unauthorized access is prohibited.

    - Configure logging synchronous for the console line.

    - Configure the IP address listed in the Addressing Table for ALL interfaces except for the loopback interface on R2.

Set the clock rate for all DCE serial interfaces at 128000. E.g.,

R1(config)# int se0/0/0

R1(config-if)# ip add 192.168.12.1 255.255.255.252

R1(config-if)# clock rate 128000

R1(config-if)# no shut

    - Copy the running configuration to the startup configuration.

  - Configure PC hosts.

  - Test connectivity.

The routers should be able to ping one another from the direct links, and each PC should be able to ping its default gateway.

The PCs are unable to ping other PCs until OSPF routing is configured. Verify and troubleshoot if necessary.

- Configure and Verify OSPF Routing

In Part 2, you will configure OSPFv2 routing on all routers in the network and then verify that routing tables are updated correctly. After OSPF has been verified, you will configure OSPF authentication on the links for added security.

  - Configure OSPF on R1.

    - Use the router ospf command in global configuration mode to enable OSPF on R1.

R1(config)# router ospf 1

Note: The OSPF process id is kept locally and has no meaning to other routers on the network.

    - Configure the network statements for the networks on R1. Use an area ID of 0.

R1(config-router)# network 192.168.1.0 0.0.0.255 area 0

R1(config-router)# network 192.168.12.0 0.0.0.3 area 0

R1(config-router)# network 192.168.13.0 0.0.0.3 area 0

  - Configure OSPF on R2 and R3.

Use the router ospf command and add the network statements for the networks on R2 and R3. On R2, do NOT advertise the Loopback 0 interface/network to OSPF.

Neighbor adjacency messages display on R1 when OSPF routing is configured on R2 and R3.

R1#

00:22:29: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.23.1 on Serial0/0/0 from LOADING to FULL, Loading Done

R1#

00:23:14: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.23.2 on FastEthernet0/1 from LOADING to FULL, Loading Done

R1#

Similar neighbor adjacency messages display on R2 and R3 as the OSPF routing is configured on the other routers.

  - Verify OSPF neighbors and routing information.

    - Issue the show ip ospf neighbor command to verify that each router lists the other routers in the network as neighbors.

R1# show ip ospf neighbor

Neighbor ID      Pri   State           Dead Time   Address         Interface

192.168.23.2       0   FULL/  -        00:00:33    192.168.13.2    FastEthernet0/1

192.168.12.2      0   FULL/  -        00:00:30    192.168.12.2    Serial0/0/0

    - Issue the show ip route command to verify that all networks display in the routing table on all routers.

R1# show ip route

<Output omitted>

Gateway of last resort is not set

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C       192.168.1.0/24 is directly connected, FastEthernet0/0

O    192.168.2.0/24 [110/65] via 192.168.12.2, 00:32:33, Serial0/0/0

O    192.168.3.0/24 [110/2] via 192.168.13.2, 00:31:48, FastEthernet0/1

192.168.12.0/24 is subnetted, 1 subnets

C       192.168.12.0/30 is directly connected, Serial0/0/0

192.168.13.0/24 is subnetted, 1 subnets

C       192.168.13.0/30 is directly connected, FastEthernet0/1

192.168.23.0/30 is subnetted, 1 subnets

O       192.168.23.0/30 [110/65] via 192.168.13.2, 00:31:38, FastEthernet0/1

What command would you use to only see the OSPF routes in the routing table?

_______________________________________________________________________________________

  - Verify OSPF protocol settings.

The show ip protocols command is a quick way to verify vital OSPF configuration information. This information includes the OSPF process ID, the router ID, networks the router is advertising, the neighbors the router is receiving updates from, and the default administrative distance, which is 110 for OSPF.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 192.168.13.1

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 0

192.168.12.0 0.0.0.3 area 0

192.168.13.0 0.0.0.3 area 0

Routing Information Sources:

Gateway         Distance      Last Update

192.168.23.2         110      00:19:16

192.168.23.1         110      00:20:03

Distance: (default is 110)

  - Verify OSPF process information.

Use the show ip ospf command to examine the OSPF process ID and router ID. This command displays the OSPF area information, as well as the last time the Shortest Path First Algorithm, i.e. the SPF algorithm, was calculated.

R1# show ip ospf

Routing Process "ospf 1" with ID 192.168.13.1

Start time: 00:20:23.260, Time elapsed: 00:25:08.296

Supports only single TOS(TOS0) routes

Supports opaque LSA

Supports Link-local Signaling (LLS)

Supports area transit capability

Supports NSSA (compatible with RFC 3101)

Event-log enabled, Maximum number of events: 1000, Mode: cyclic

Router is not originating router-LSAs with maximum metric

Initial SPF schedule delay 5000 msecs

Minimum hold time between two consecutive SPFs 10000 msecs

Maximum wait time between two consecutive SPFs 10000 msecs

Incremental-SPF disabled

Minimum LSA interval 5 secs

Minimum LSA arrival 1000 msecs

LSA group pacing timer 240 secs

Interface flood pacing timer 33 msecs

Retransmission pacing timer 66 msecs

Number of external LSA 0. Checksum Sum 0x000000

Number of opaque AS LSA 0. Checksum Sum 0x000000

Number of DCbitless external and opaque AS LSA 0

Number of DoNotAge external and opaque AS LSA 0

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Number of areas transit capable is 0

External flood list length 0

IETF NSF helper support enabled

Cisco NSF helper support enabled

Reference bandwidth unit is 100 mbps

Area BACKBONE(0)

Number of interfaces in this area is 3

Area has no authentication

SPF algorithm last executed 00:22:53.756 ago

SPF algorithm executed 7 times

Area ranges are

Number of LSA 3. Checksum Sum 0x019A61

Number of opaque link LSA 0. Checksum Sum 0x000000

Number of DCbitless LSA 0

Number of indication LSA 0

Number of DoNotAge LSA 0

Flood list length 0

  - Verify OSPF interface settings.

    - Issue the show ip ospf interface brief command to display a summary of the OSPF-enabled interfaces.

R1# show ip ospf interface brief

Interface    PID   Area            IP Address/Mask    Cost  State Nbrs F/C

Fa0/0        1     0               192.168.1.1/24     1     DR    0/0

Fa0/1        1     0               192.168.13.1/30    1     DR    1/1

Se0/0/0      1     0               192.168.12.1/30    64    P2P   1/1

    - For a more detailed list of every OSPF-enabled interface, issue the show ip ospf interface command.

R1# show ip ospf interface

FastEthernet0/0 is up, line protocol is up

Internet Address 192.168.1.1/24, Area 0, Attached via Network Statement

Process ID 1, Router ID 192.168.13.1, Network Type BROADCAST, Cost: 1

Topology-MTID    Cost    Disabled    Shutdown      Topology Name

0           1         no          no            Base

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 192.168.13.1, Interface address 192.168.1.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:01

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 0, maximum is 0

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

Serial0/0/0 is up, line protocol is up

Internet Address 192.168.12.1/30, Area 0, Attached via Network Statement

Process ID 1, Router ID 192.168.13.1, Network Type POINT_TO_POINT, Cost: 64

Topology-MTID    Cost    Disabled    Shutdown      Topology Name

0           64        no          no            Base

Transmit Delay is 1 sec, State POINT_TO_POINT

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:03

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 192.168.23.1

Suppress hello for 0 neighbor(s)

FastEthernet0/1 is up, line protocol is up

Internet Address 192.168.13.1/30, Area 0, Attached via Network Statement

Process ID 1, Router ID 192.168.13.1, Network Type POINT_TO_POINT, Cost: 64

Topology-MTID    Cost    Disabled    Shutdown      Topology Name

0           64        no          no            Base

Transmit Delay is 1 sec, State POINT_TO_POINT

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:01

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 3/3, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 192.168.23.2

Suppress hello for 0 neighbor(s) FastEthernet

  - Verify end-to-end connectivity.

Each PC should be able to ping the other PCs in the topology. Verify and troubleshoot if necessary.

Note: It may be necessary to disable the PC firewall to ping between PCs.

- Configure and Redistribute a Default Route

  - Configure a loopback address on the R2 router to simulate a link to an ISP.

R2(config)# interface loopback0

R2(config-if)#ip address 209.165.201.2 255.255.255.255

  - Configure a static default route on the R2 router.

Use the loopback interface that has been configured to simulate a link to an ISP as the exit
interface. This forwards any unknown destination address traffic to the R2 L0, simulating the Internet by setting a Gateway of Last Resort on the R2 router.

R2(config)# ip route 0.0.0.0 0.0.0.0 lo0

NOTE: While using a next-hop address as the target of a static default route, is often preferable to using the name of the exit interface.  However, here, where we are using a Loopback interface, there is no ‘next hop address’.  So we must use the exit interface.

  - Use the default-information originate command to include the default route in the OSPF updates that are sent from the R2 router.

R2(config)# router ospf 1

R2(config-router)# default-information originate

  - View the routing table on R1 and R3 to verify that the static default route is being redistributed via OSPF.

    - View the routing table on R1 to verify that the static default route is being redistributed via OSPF.

R1# show ip route

<Output omitted>

Gateway of last resort is 192.168.12.2 to network 0.0.0.0

C    192.168.1.0/24 is directly connected, FastEthernet0/0

O    192.168.2.0/24 [110/65] via 192.168.12.2, 00:17:30, Serial0/0/0

O    192.168.3.0/24 [110/2] via 192.168.13.2, 00:16:13, FastEthernet0/1

192.168.12.0/30 is subnetted, 1 subnets

C       192.168.12.0 is directly connected, Serial0/0/0

192.168.13.0/30 is subnetted, 1 subnets

C       192.168.13.0 is directly connected, FastEthernet0/1

192.168.23.0/30 is subnetted, 1 subnets

O       192.168.23.0 [110/65] via 192.168.13.2, 00:16:13, FastEthernet0/1

O*E2 0.0.0.0/0 [110/1] via 192.168.12.2, 00:00:06, Serial0/0/0

How can you tell from the routing table that the subnetted network shared by R1 has a pathway for Internet traffic?

____________________________________________________________________________________

How can you tell that R1 has learnt about this route from OSPF?

____________________________________________________________________________________

What is the value of the metric for this route? Why it have the value that it does?

____________________________________________________________________________________

    - ____________________________________________________________________________________ View the routing table on R3 to verify that the static default route is being redistributed via OSPF.

How is the pathway for Internet traffic  get into its routing table?

____________________________________________________________________________________

  - Verify connectivity.

    - Simulate sending traffic to the Internet by pinging from PC-A and PC-C to 209.165.201.2.

Were the pings successful? _________

    - Verify that hosts within the subnetted network can reach each other by pinging between PC-A and PC-C.

Were the pings successful? _________

Note: It may be necessary to disable the PCs firewall.

    - Verify the route that is taken for the packets from PC-A to PC-C using tracert command. Which route did the packets take? Explain why. _____________________________________________

- Change Router ID Assignments

The OSPF router ID is used to uniquely identify the router in the OSPF routing domain. Cisco routers derive the router ID in one of three ways and with the following precedence:

      - IP address configured with the OSPF router-id command, if present

      - Highest IP address of any of the router’s loopback addresses, if present

      - Highest active IP address on any of the router’s physical interfaces

Because no router IDs or loopback interfaces have been configured on the three routers, the router ID for each router is determined by the highest IP address of any active interface.

In this Part, you will change the OSPF router ID assignment using loopback addresses. You will also use the router-id command to change the router ID.

  - Change router IDs using loopback addresses.

    - Assign an IP address to loopback 1 on R1.

R1(config)# interface lo0

R1(config-if)# ip address 1.1.1.1 255.255.255.255

R1(config-if)# end

    - Assign IP addresses to Loopback 1 on R2 and R3. Use IP address 2.2.2.2/32 for R2 and 3.3.3.3/32 for R3.

    - You must either reload the router or use the clear ip ospf process command in order to reset the router ID to the loopback address. Issue the clear ip ospf process command on all three routers. Type yes to reply to the reset verification message, and press ENTER.

    - Issue the show ip protocols command to view the new router ID.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 1.1.1.1

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 0

192.168.12.0 0.0.0.3 area 0

192.168.13.0 0.0.0.3 area 0

Routing Information Sources:

Gateway         Distance      Last Update

3.3.3.3              110      00:01:00

2.2.2.2              110      00:01:14

Distance: (default is 110)

    - Issue the show ip ospf neighbor command to display the router ID changes for the neighboring routers.

R1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

3.3.3.3           0   FULL/DR         00:00:35    192.168.13.2    FastEthernet0/1

2.2.2.2           0   FULL/  -        00:00:32    192.168.12.2    Serial0/0/0

R1#

  - Change the router ID on R1 using the router-id command.

The preferred method for setting the router ID is with the router-id command.

    - Issue the router-id 11.11.11.11 command on R1 to reassign the router ID. Notice the informational message that appears when issuing the router-id command.

R1(config)# router ospf 1

R1(config-router)# router-id 11.11.11.11

Reload or use "clear ip ospf process" command, for this to take effect

R1(config)# end

    - You will receive an informational message telling you that you must either reload the router or use the clear ip ospf process command for the change to take effect. Issue the clear ip ospf process command on all three routers. Type yes to reply to the reset verification message, and press ENTER.

    - Set the router ID for R2 to 22.22.22.22 and the router ID for R3 to 33.33.33.33. Then use clear ip ospf process command to reset ospf routing process.

    - Issue the show ip protocols command to verify that the router ID changed on R1.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 11.11.11.11

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 0

192.168.12.0 0.0.0.3 area 0

192.168.13.0 0.0.0.3 area 0

Passive Interface(s):

FastEthernet0/1

Routing Information Sources:

Gateway         Distance      Last Update

33.33.33.33          110      00:00:19

22.22.22.22          110      00:00:31

3.3.3.3              110      00:00:41

2.2.2.2              110      00:00:41

Distance: (default is 110)

    - Issue the show ip ospf neighbor command on R1 to verify that new router ID for R2 and R3 is listed.

R1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

33.33.33.33       0   FULL/DR         00:00:36    192.168.13.2    FastEthernet0/1

22.22.22.22       0   FULL/  -        00:00:32    192.168.12.2    Serial0/0/0

- Configure OSPF Passive Interfaces

The passive-interface command prevents routing updates from being sent through the specified router interface. This is commonly done to reduce traffic on the LANs because they do not need to receive dynamic routing protocol communication.

In this part, you will use the passive-interface command to configure a single interface as passive. You will also configure OSPF so that all interfaces on the router are passive by default, and then enable OSPF routing advertisements on selected interfaces.

  - Configure a passive interface.

    - Issue the show ip ospf interface Fa0/0 command on R1. Notice the timer indicating when the next Hello packet is expected. Hello packets are sent every 10 seconds and are used between OSPF routers to verify that their neighbors are up.

R1# show ip ospf interface Fa0/0

FastEthernet0/0 is up, line protocol is up

Internet Address 192.168.1.1/24, Area 0, Attached via Network Statement

Process ID 1, Router ID 11.11.11.11, Network Type BROADCAST, Cost: 1

Topology-MTID    Cost    Disabled    Shutdown      Topology Name

0           1         no          no            Base

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 11.11.11.11, Interface address 192.168.1.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:02

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 0, maximum is 0

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

    - Issue the passive-interface command to change the Fa0/0 interface on R1 to passive.

R1(config)# router ospf 1

R1(config-router)# passive-interface Fa0/0

    - Re-issue the show ip ospf interface g0/0 command to verify that Fa0/0 is now passive.

R1# show ip ospf interface Fa0/0

FastEthernet0/0 is up, line protocol is up

Internet Address 192.168.1.1/24, Area 0, Attached via Network Statement

Process ID 1, Router ID 11.11.11.11, Network Type BROADCAST, Cost: 1

Topology-MTID    Cost    Disabled    Shutdown      Topology Name

0           1         no          no            Base

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 11.11.11.11, Interface address 192.168.1.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

No Hellos (Passive interface)

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 0, maximum is 0

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

    - Issue the show ip route command on R2 and R3 to verify that a route to the 192.168.1.0/24 network is still available.

R2# show ip route

<output omitted>

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

2.0.0.0/32 is subnetted, 1 subnets

C        2.2.2.2 is directly connected, Loopback0

O     192.168.1.0/24 [110/65] via 192.168.12.1, 00:58:32, Serial0/0/0

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/24 is directly connected, FastEthernet0/0

O     192.168.3.0/24 [110/65] via 192.168.23.2, 00:58:19, Serial0/0/1

192.168.12.0/24 is subnetted, 1 subnets

C        192.168.12.0/30 is directly connected, Serial0/0/0

192.168.13.0/30 is subnetted, 1 subnets

O        192.168.13.0 [110/128] via 192.168.23.2, 00:58:19, Serial0/0/1

[110/128] via 192.168.12.1, 00:58:32, Serial0/0/0

192.168.23.0/24 is subnetted, 1 subnets

C        192.168.23.0/30 is directly connected, Serial0/0/1

S* 0.0.0.0/0 is directly connected, Loopback0

  - Set passive interface as the default on a router.

    - Issue the show ip ospf neighbor command on R1 to verify that R2 is listed as an OSPF neighbor.

R1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

33.33.33.33       0   FULL/DR         00:00:31    192.168.13.2    FastEthernet0/1

22.22.22.22       0   FULL/  -        00:00:32    192.168.12.2    Serial0/0/0

    - Issue the passive-interface default command on R2 to set the default for all OSPF interfaces as passive.

R2(config)# router ospf 1

R2(config-router)# passive-interface default

R2(config-router)#

*Apr  3 00:03:00.979: %OSPF-5-ADJCHG: Process 1, Nbr 11.11.11.11 on Serial0/0/0 from FULL to DOWN, Neighbor Down: Interface down or detached

*Apr  3 00:03:00.979: %OSPF-5-ADJCHG: Process 1, Nbr 33.33.33.33 on Serial0/0/1 from FULL to DOWN, Neighbor Down: Interface down or detached

    - Re-issue the show ip ospf neighbor command on R1. After the dead timer expires, R2 will no longer be listed as an OSPF neighbor.

R1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

33.33.33.33       0   FULL/  -        00:00:34    192.168.13.2    Serial0/0/1

    - Issue the show ip ospf interface S0/0/0 command on R2 to view the OSPF status of interface S0/0/0.

R2# show ip ospf interface s0/0/0

Serial0/0/0 is up, line protocol is up

Internet Address 192.168.12.2/30, Area 0, Attached via Network Statement

Process ID 1, Router ID 22.22.22.22, Network Type POINT_TO_POINT, Cost: 64

Topology-MTID    Cost    Disabled    Shutdown      Topology Name

0           64        no          no            Base

Transmit Delay is 1 sec, State POINT_TO_POINT

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

No Hellos (Passive interface)

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 0, maximum is 0

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

    - If all interfaces on R2 are passive, then no routing information is being advertised. In this case, R1 and R3 should no longer have a route to the 192.168.2.0/24 network. You can verify this by using the show ip route command.

    - On R2, issue the no passive-interface command so the router will send and receive OSPF routing updates. After entering this command, you will see an informational message that a neighbor adjacency has been established with R1.

R2(config)# router ospf 1

R2(config-router)# no passive-interface s0/0/0

R2(config-router)#

*Apr  3 00:18:03.463: %OSPF-5-ADJCHG: Process 1, Nbr 11.11.11.11 on Serial0/0/0 from LOADING to FULL, Loading Done

    - Re-issue the show ip route and show ip ospf neighbor commands on R1 and R3, and look for a route to the 192.168.2.0/24 network.

Which interface is R3 using to route to the 192.168.2.0/24 network? ____________

What is the accumulated cost metric for the 192.168.2.0/24 network on R3? _________

Does R2 show up as an OSPF neighbor on R1? ________

Does R2 show up as an OSPF neighbor on R3? ________

What does this information tell you?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

    - Change interface S0/0/1 on R2 to allow it to advertise OSPF routes. Record the commands used below.

____________________________________________________________________________________

____________________________________________________________________________________

    - Re-issue the show ip route command on R3.

Which interface is R3 using to route to the 192.168.2.0/24 network? ____________

What is the accumulated cost metric for the 192.168.2.0/24 network on R3 now and how is this calculated?

____________________________________________________________________________________

Is R2 listed as an OSPF neighbor to R3? ________

- Reflection

  - Why is it important to control the router ID assignment when using the OSPF protocol?

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

  - Why would you want to set an OSPF interface to passive?

_______________________________________________________________________________________

_______________________________________________________________________________________